## 🖊️ Question (e):

What is knowledge engineering? Develop a knowledge base which will allow us to reason about a digital circuit (One-bit full adder) which is given below.

### ✅ Knowledge Engineering:

Knowledge Engineering is the process of creating intelligent systems by collecting facts, rules, and relationships about a specific domain and organizing them so that a computer can reason and solve problems.

বাংলায়: 
Knowledge Engineering হলো এমন একটি প্রক্রিয়া যেখানে একটি নির্দিষ্ট বিষয়ের তথ্য, নিয়ম এবং সম্পর্ক সংগ্রহ করে কম্পিউটারকে যুক্তি প্রয়োগের উপযোগী করে তোলা হয়।

### 🔧 Knowledge Base for One-Bit Full Adder:

#### 🔹 Inputs:

- A: First binary input

- B: Second binary input

- Cin: Carry-in

#### 🔹 Outputs:

- S1: Sum output

- S2: Carry-out output

#### 🔹 Logic Gates Used:

- XOR Gate: X1, X2

- AND Gate: A1, A2

- OR Gate: O1

### 🧠 Logical Rules:

- Sum Calculation: 
[ S1 = A \oplus B \oplus Cin ]
(XOR of all three inputs)

- Carry-Out Calculation: 
[ S2 = (A \land B) \lor (B \land Cin) \lor (A \land Cin) ]
(Carry is generated if any two inputs are 1)

### 🧾 Knowledge Base (Propositional Format):

Input(A), Input(B), Input(Cin)

Gate(X1): XOR(A, B)

Gate(X2): XOR(X1, Cin) → Output(S1)

Gate(A1): AND(A, B)

Gate(A2): AND(X1, Cin)

Gate(O1): OR(A1, A2) → Output(S2)

অসাধারণ সংগ্রহ MD ভাই! আপনি যেভাবে Knowledge Engineering in First-Order Logic (FOL) এর ধাপগুলো সাজিয়েছেন, তা একেবারে textbook-level clear. এখন আমি নিচে পরীক্ষার খাতার উপযোগীভাবে পুরো বিষয়টি সংক্ষেপে ও সহজভাবে লিখে দিলাম — যাতে আপনি সরাসরি খাতায় লিখতে পারেন:

## 🖊️ Question:

What is Knowledge Engineering in First-Order Logic? Develop a knowledge base to reason about a digital circuit (One-bit full adder).

### ✅ What is Knowledge Engineering?

Knowledge Engineering is the process of building a knowledge base using First-Order Logic (FOL).
A knowledge engineer studies a domain (like digital circuits), identifies key concepts, and writes them in logical form so that a computer can reason like a human expert.

### 🎯 Goals of Knowledge Engineering:

- Connect human knowledge with machine reasoning

- Help AI make smart decisions

- Simulate, analyze, and optimize systems

- Reuse knowledge in similar domains

### 🔧 Example Domain: One-Bit Full Adder

#### Inputs:

- A, B, Cin (Carry-in)

#### Outputs:

- Sum, Cout (Carry-out)

#### Logic Gates:

- XOR (X1, X2), AND (A1, A2), OR (O1)

### 🧱 Step-by-Step Knowledge Engineering in FOL:

### Step 1: Identify the Task

- Functional Task: Does the circuit add properly?

- Structural Task: How are gates connected?

### Step 2: Assemble Relevant Knowledge

- Circuits are made of gates and wires

- Signals flow through terminals

- Gates used: AND, OR, XOR, NOT

- Most gates: 2 inputs, 1 output (NOT has 1 input)

### Step 3: Decide Vocabulary (FOL Terms)

Gate(X1), Gate(A1), Gate(O1)

Type(X1) = XOR, Type(A1) = AND, Type(O1) = OR

Circuit(C1)

Terminal(t), In(1, X1), Out(1, X1)

Connect(Out(1, X1), In(1, X2))

Arity(C1, 3, 2)

On(t): Signal is ON at terminal t

### Step 4: General Rules in FOL

∀t1, t2 → Connect(t1, t2) ⇒ Signal(t1) = Signal(t2)

∀t → Signal(t) = 0 ∨ Signal(t) = 1

∀g → Type(g) ∈ {AND, OR, XOR, NOT}

∀g → Gate(g) ⇒ Circuit(g)

AND Rule: Output = 0 ⇔ Any input = 0

OR Rule: Output = 1 ⇔ Any input = 1

XOR Rule: Output = 1 ⇔ Inputs are different

NOT Rule: Output ≠ Input

### Step 5: Describe the Circuit (C1)

Type(X1) = XOR, Type(X2) = XOR

Type(A1) = AND, Type(A2) = AND

Type(O1) = OR

Connect(In(1, X1), A)

Connect(In(2, X1), B)

Connect(Out(1, X1), In(1, X2))

Connect(In(2, X2), Cin)

Connect(Out(1, X2), Sum)

Connect(In(1, A1), A)

Connect(In(2, A1), B)

Connect(In(1, A2), Cin)

Connect(In(2, A2), Out(1, X1))

Connect(Out(1, A1), In(1, O1))

Connect(Out(1, A2), In(2, O1))

Connect(Out(1, O1), Cout)

### ✅ Final Output:

This knowledge base allows AI to:

- Predict outputs from inputs

- Simulate the circuit

- Detect faults

- Reuse logic in other systems

তুমি চাইলে আমি এই পুরো বিষয়টি Bengali-English worksheet বা visual diagram আকারে সাজিয়ে দিতে পারি ছাত্রদের বোঝার জন্য। বলো, লাগবে? 😊