1. Give some advantages and disadvantages of the connectionless service.

2. Give some advantages and disadvantages of the connection-oriented service

## ✅ Summary Table

| Service Type | Advantages | Disadvantages |
| --- | --- | --- |
| Connectionless | Fast, low overhead, simple | No reliability, no order, no control |
| Connection-Oriented | Reliable, ordered, controlled | Slower, more overhead, complex |

3. If a label in a connection-oriented service is n bits, how many virtual circuits can

be established at the same time?

## ✅ Step-by-Step Explanation (Bangla + English Mixed)

### 🔷 Label in connection-oriented service:

- প্রতিটি virtual circuit-এর জন্য একটি label ব্যবহার করা হয়

- যদি label size হয় n bits, তাহলে total possible label combinations = (2^n)

### ✅ Final Answer:

Number of virtual circuits = 2n

বাংলা ব্যাখ্যা: যেহেতু n-bit label দিয়ে প্রতিটি circuit আলাদা করা হয়, তাই মোট 2nটি unique label তৈরি করা যায় → অর্থাৎ একসাথে 2nটি virtual circuit স্থাপন করা সম্ভব।

## ✅ Example:

- If label size = 12 bits

- → Virtual circuits = 212=4096

4. Assume a destination computer receives messages from several computers. How can it be sure that the fragments from one source is not mixed with the fragments from another source.

### 🔷 Problem:

Destination computer receives messages from multiple sources. → কিভাবে নিশ্চিত হবে যে এক source-এর fragment অন্য source-এর সঙ্গে মিশে যাচ্ছে না?

## ✅ Answer:

To prevent mixing of fragments from different sources, each fragment carries identification information.

### 🔹 Key Mechanism:

- Source IP address: Identifies the sender → প্রতিটি fragment-এর header-এ source IP থাকে

- Destination IP address: Identifies the receiver

- Protocol field: Indicates the upper-layer protocol (e.g., TCP, UDP)

- Identification field (in IP header): → Same for all fragments of one datagram → Destination uses this to reassemble fragments correctly

5. Assume a destination computer receives several packets from a source. How can it be sure that the fragments belonging to a datagram are not mixed from the frag- ments belonging to another datagram.

## ✅ Final Answer:

Fragments are grouped using the Identification field in the IP header, along with Source and Destination IP addresses. This ensures that fragments from different datagrams are not mixed.

বাংলা ব্যাখ্যা: Destination computer IP header-এর Identification field, Source IP, এবং Destination IP ব্যবহার করে fragment গুলোকে সঠিকভাবে আলাদা করে এবং একই datagram-এর অংশগুলো একত্র করে।

## 7. Compare and contrast the delays in connectionless and connection-oriented ser-vices. Which service creates less delay if the message is large? Which service cre-ates less delay if the message is small?

## ✅ Delay Comparison: Connectionless vs Connection-Oriented

| Aspect | Connectionless Service | Connection-Oriented Service |
| --- | --- | --- |
| Initial Setup Delay | No setup delay | Requires setup (3-way handshake in TCP) |
| Per-Packet Delay | Each packet sent independently | Packets follow established path |
| Reliability Overhead | No retransmission or ACK | Retransmission, ACK, flow control add delay |
| Ordering Delay | No guarantee of order | Packets arrive in order |

## ✅ Delay Based on Message Size

### 🔷 If message is small:

- Connectionless service creates less delay 
→ No setup, no ACK, fast delivery
→ ছোট message দ্রুত পাঠানো যায়

### 🔷 If message is large:

- Connection-oriented service creates less delay overall 
→ Once connection is set, packets flow efficiently
→ Reliable delivery ensures fewer retransmissions

## ✅ Final Summary

| Message Size | Less Delay Service | Explanation |
| --- | --- | --- |
| Small | Connectionless | No setup, fast transmission |
| Large | Connection-Oriented | Efficient flow after setup |